ASL Dual-Scale Macro Ring

Full Z-Matrix Verification Memo v0.1

GCF–PKG-E / ASL Platform | June 2026 | PROVISIONAL

Formal status: PROVISIONAL WORKING WINDOW CONFIRMED — NOT a robustness PASS. Monte Carlo gate probability 70–84% (threshold 90% not met). Variance reduction metric numerically invalid in MC. Phase tolerance ±10° achieved only at the conservative 1–2%/75° window.

# 1. Purpose

This memo records the results of full Z-matrix coupled-circuit verification of the macro outer ring in the ASL dual-scale field architecture. It supersedes the simplified linear-superposition scan (narrow scan v0.1) and corrects the 'central boost reduction = 100%' artefact that arose from that simplified model.

# 2. Architecture Context

The dual-scale ring field architecture comprises two layers:

- Layer 0 — Outer macro ring: large square ring enclosing the full package footprint. Role: weak global bias, calibration support, slow drift compensation.

- Layer 1 — Inner five rings: four regional rings (rA–rD) and one central ring (rZ). Role: regional field Tx, emergency boost, sensing.

The macro ring is NOT a primary field sustainer. It is a low-current, phase-controlled bias/calibration loop that operates alongside the inner five rings without replacing any of their functions.

# 3. Verification Method

| Parameter | Value | Source |
| --- | --- | --- |
| Z-matrix | 11×11 full coupled-circuit | FastHenry Phase 2 extraction (macro + 5 inner + 4 Rx + SX) |
| Solve method | Complete Z-matrix (same as Phase 2c-R2) | Coupled-circuit, not linear superposition |
| Frequency | 1 GHz | Operating frequency |
| Gate: V_sx | V_sx/V_sx0 ≤ 1.08 | Tightened from 1.10 for manufacturing margin |
| Gate: variance | Fault Rx variance reduction ≥ 10% | Full matrix, 30% fault on rA |
| Gate: boost | Central I_Z reduction ≥ 10% | 30% fault → R_rx = 0.90 target |
| Gate: phase tol | Gates hold at ±5° phase perturbation | Robustness criterion |
| MC | N = 300, phase ±10°, frac ±15%, I_reg ±3% | Gate probability target: ≥ 90% |

# 4. Anchor Point Results

| frac | Phase | V_sx ratio | V_rx_min ratio | Gate | ±5° robust | ±10° robust |
| --- | --- | --- | --- | --- | --- | --- |
| 1% | 75° | 1.027 | 1.030 | PASS ✓ | ✓ | ✓ |
| 1% | 90° | 1.004 | 1.005 | FAIL ✗ | ✗ | ✗ |
| 2% | 75° | 1.061 | 1.068 | PASS ✓ | ✓ | ✗ |
| 2% | 90° | 1.016 | 1.019 | PASS ✓ | fragile ✗ | ✗ |
| 3% | 90° | 1.036 | 1.043 | PASS ✓ | ✓ | ✗ |
| 3% | 105° | 0.966 | 0.967 | PASS ✓ | ✓ | ✓ |
| 5% | 90° | 1.097 | 1.114 | FAIL ✗ | ✗ | ✗ |
| 5% | 105° | 0.985 | 0.993 | PASS ✓ | ✓ | ✓ |

Key corrected result vs simplified scan: baseline I_Z needed is 10.3 mA (not 100 mA). The simplified model used different normalization. Central boost reduction values in this memo are relative to the 10.3 mA full-matrix baseline.

# 5. Preferred Operating Windows

## 5.1 Conservative / most robust

| Parameter | Specification |
| --- | --- |
| Current fraction | 1–2% × I_regional |
| Phase | 75° (quadrature-leading) |
| Phase tolerance | ±10° (robust) |
| V_sx ratio | 1.03–1.06 (margin ≤0.05 below gate) |
| V_rx_min change | +3–7% (slight boost to receivers) |
| P_macro | 0.008–0.033 mW |
| Primary gate mechanism | Boost reduction (central I_Z reduced) |

## 5.2 Higher benefit, narrower phase window

| Parameter | Specification |
| --- | --- |
| Current fraction | 3% × I_regional |
| Phase | 90°–105° |
| Phase tolerance | ±5° at 105°; fragile at 90° |
| V_sx ratio | 0.97–1.04 (105° reduces SX below baseline) |
| V_rx_min change | −4–44% depending on phase |
| P_macro | 0.128 mW |
| Primary gate mechanism | Both variance reduction and boost reduction |

# 6. Monte Carlo Results

| Configuration | P(gate) | V_sx p90 | Boost red. mean | Status |
| --- | --- | --- | --- | --- |
| 2% / 90°, MC N=300 | 70.3% | 1.045 ✓ | +32.2% | Below 90% threshold |
| 3% / 90°, MC N=300 | 83.7% | 1.076 ✓ | +63.7% | Below 90% threshold |

Variance reduction metric produced extreme negative values (−10,000%) in Monte Carlo. This is a numerical instability in the current normalization under phase perturbation. The variance metric is INVALID for MC reporting and must be repaired before use. P(gate) figures above are based on boost reduction and V_sx gates only.

# 7. Rejected Operating Modes

| Mode | Reason |
| --- | --- |
| Continuous in-phase (0°) background field | V_sx immediately exceeds 1.10 at any current fraction |
| Counter-current (180°) | Suppresses receiver field; harmful |
| 5% / 90° | V_sx = 1.097, margin < 0.003; not manufacturing-safe |
| 2% / 90° as primary point | Passes nominally but fragile: ±5° fails |
| Macro as emergency boost actor | Not its role; central ring rZ handles boost |

# 8. Safe Claim

A weak, phase-controlled macro ring (1–3% of regional current, 75°–105° phase) can provide a viable global bias and calibration contribution to the inner five-ring ASL architecture, with partial central boost reduction and sensitive-zone ratio within 1.08. Robust manufacturing-level stability (Monte Carlo ≥ 90%) has not yet been established.

# 9. Next Steps (before robustness upgrade)

- Repair Monte Carlo variance normalization (numerical instability under phase perturbation)

- Run joint geometric + phase + return-path perturbation MC at 1–2%/75° and 3%/90–1 05°

- If P(gate) ≥ 90% after repair: upgrade to ROBUSTNESS PASS

- If P(gate) < 90% after repair: retain macro ring as calibration-only auxiliary, remove active bias/sustainment claim

ASL Dual-Scale Macro Ring Memo v0.1 — Confidential — PROVISIONAL WORKING WINDOW CONFIRMED | Not a robustness PASS