PKG-E Active Sustainment Loop (ASL)

Formal Closure Document — v0.1

Post Phase 2c-R2 | Full-Matrix Validated | June 2026

Evidence status: All results derived from FastHenry magneto-quasistatic extraction, coupled-circuit frequency-domain models, and normalized control simulations. No silicon measurement, real package parasitics, BER, or driver circuit verification. Consistency gate: dynamic→static asymptotic error < 0.001%.

# 1. Core Statement

ASL is not a full-fault-range self-healing system. It is an active boost layer that maintains the primary field structure under up to 30% outer-ring current degradation, and provides degraded-survival mitigation for 40–50% degradation.

# 2. Evidence Chain Status

| Phase | Component | Status | Notes |
| --- | --- | --- | --- |
| A4b | FastHenry 5×5 matrix extraction | VERIFIED | r=1.0mm central, r=1.5mm outer, all geometry consistent |
| Phase 2 | Static coupling baseline | VERIFIED | M_central→outer: 108.9–109.2 pH symmetric |
| Phase 2c-R2 | Dynamic→static consistency | PASS ≤ 0.001% | Full Z-matrix used; simplified model superseded |
| Phase 2b | Current-duty thermal proxy | CORRECTED | Static compensation window; rise time not bottleneck |
| Phase 2 | Recovery success 100% | WITHDRAWN | Was post-fault metric, not fault-period sustainment |
| Text | 20µs pulse wording | CORRECTED TO 20ns | Code used ns throughout |
| Zone Audit v0.2 | Net area (neutral scenario) | PROVISIONAL | +4.67mm²; pessimistic scenario negative; dedup pending |

# 3. Formal Working Window

## 3.1 Full sustainment (G1: R_rx ≥ 0.90)

| Fault severity | Central current | R_rx | Sensitive ratio | Status | Driver note |
| --- | --- | --- | --- | --- | --- |
| 20% outer degradation | 50 mA | 0.9224 | 1.028 | FULL SUSTAINMENT ✓ | V_supply≤0.4V, P≤5mW |
| 30% outer degradation | 100 mA | 0.9110 | 1.071 | FULL SUSTAINMENT ✓ | Preferred operating point |
| 30% margin mode | 125 mA | 0.9391 | 1.099 | FULL SUSTAINMENT ✓ | Warning: sensitive margin ≈0.001 exhausted |

Driver clarification for 100mA recommended point:

- V_supply limit = 0.4 V

- V_driver_resistive_drop ≈ 0.20 V (at R_drv = 2Ω)

- P_driver_resistive = I² × R_drv = 0.100² × 2 = 20 mW (peak, short pulse)

- Average power at 5% duty cycle ≈ 1.0 mW << 8.1 mW Phase C2 saving

## 3.2 Boundary and degraded regime

| Fault severity | Central current | R_rx | Sensitive ratio | Status |
| --- | --- | --- | --- | --- |
| 40% outer degradation | 150 mA | 0.8996 | 1.113 ✗ exceeds 1.10 | DEGRADED SURVIVAL ONLY |
| 50% outer degradation | ≤150 mA | < 0.85 | exceeds limit | DEGRADATION MITIGATION ONLY |

40% fault is NOT borderline rounding. Both R_rx < 0.90 and sensitive ratio > 1.10 simultaneously. Raising current would push sensitive zone further over limit. This is the correct stop boundary.

# 4. Central Ring Multi-Function Modes

| Mode | Current | Power | Function | Preempt to emergency |
| --- | --- | --- | --- | --- |
| Normal sensing | 0.1 mA | ≈0 mW | Field health, phase/amplitude tracking | 0 ns (already monitoring) |
| Calibration | 2 mA | ≈0 mW | Amplitude/phase cal, threshold update | 5 ns |
| Low-rate broadcast | 10 mA | ≈0 mW | Health status, epoch, thermal state, telemetry | 10 ns |
| Emergency boost | 100 mA | 20 mW peak | Field sustainment during outer ring fault | 0 ns (highest priority) |

Central ring is not idle between faults. Interior loop area (2.55 mm², 81% geometrically usable) accommodates decoupling capacitors, thermal vias, power straps, and control logic.

# 5. Corrected and Withdrawn Claims

| Claim | Original | Corrected status |
| --- | --- | --- |
| Phase 2 recovery 100% | G1/G2 PASS | WITHDRAWN — measures post-fault recovery, not fault-period sustainment |
| Simplified R_rx = 0.731 | Used in Phase 2c-R | SUPERSEDED — full matrix gives 0.911; simplified model invalid |
| 20µs pulse wording | Appeared in verbal summary | CORRECTED TO 20ns — code used ns throughout |
| Phase 2b as dynamic envelope | Named 'current-duty-thermal envelope' | CORRECTED — static compensation window; Phase 2c-R2 provides dynamic validation |
| G1 pass at I_inf=389mA | Phase 2c-R result | INVALID — exceeded I_max limit; full matrix with limit gives correct result |

# 6. Remaining Engineering Validation Required

The following must be completed before any hardware or commercial claim:

- Physical driver circuit: NMOS boost driver, gate drive, slew control, supply decoupling

- Supply integrity and ground bounce: 100mA pulse onto package PDN

- Receiver front-end: decoding circuit, sensitivity threshold, BER proxy

- Current density and electromigration rules: 100mA in r=1mm, w=0.1mm, h=0.02mm trace

- Package SI and return path: full-wave OpenEMS validation of 5-ring geometry

- Deduplicated central-zone area audit: keepout/broadcast-tree overlap quantification

# 7. Approved Summary Statement

Under validated FastHenry full-matrix models with enforced current and voltage limits, the ASL central ring provides complete field sustainment (R_rx ≥ 0.90) for up to 30% outer-ring current degradation at 100 mA, with a sensitive-zone ratio of 1.071 (≤ 1.10 gate). For 40–50% degradation, only degraded-survival mitigation is available. This result is model-supported and not silicon-verified.

ASL Formal Closure v0.1 — Confidential — Evidence: FastHenry + coupled-circuit + control toy | No silicon validation