ASL-Enabled AI Chiplet Platform

Interface and Placement Specification v0.1

GCF–PKG-E / ASL | F3 Interposer/RDL | Multi-Chiplet AI Inference Package | June 2026

Evidence status: This specification derives from FastHenry magnetostatic simulation, coupled-circuit models, and normalized system toy models. No silicon measurement, real package parasitics, BER, or driver circuit verification has been performed. Square-ring FastHenry re-extraction required before layout commitment.

# 1. Platform Definition

The ASL-Enabled AI Chiplet Platform provides a package-level control and resilience fabric that can host third-party AI compute chiplets without modifying their compute datapaths. It is not a replacement for high-speed data interconnect (UCIe, NVLink, SerDes). It operates as a parallel control fabric alongside any data fabric.

Core positioning: A package platform that hosts third-party AI chiplets and provides a low-cost, high-fan-out control broadcast and active field-sustainment fabric — independent of the main data interconnect.

| Layer | Responsibility | Technology |
| --- | --- | --- |
| Data Fabric | Tensor payload, HBM traffic, compute–memory exchange | UCIe / SerDes / proprietary D2D / photonics (future) |
| ASL Control Fabric | Broadcast, barrier, health sensing, emergency control, field sustainment | PKG-E dual-domain EM + copper fallback |
| OBS Policy Layer | Overload detection, rescue decision, degradation orchestration | GCF-OBS software + hardware interface |

# 2. Five-Zone Functional Layout

| Zone | Hosted element | Primary function | ASL role |
| --- | --- | --- | --- |
| Domain A | AI Compute Chiplet A | Inference / matrix ops | Field Tx ring rA; Rx at control entry |
| Domain B | AI Compute Chiplet B | Inference / matrix ops | Field Tx ring rB; Rx at control entry |
| Domain C | HBM / Memory Control Die | Data supply and memory management | Field Tx ring rC; Rx at control entry |
| Domain D | I/O / System Control Die | External interface, telemetry, management | Field Tx ring rD; Rx at control entry |
| Central Zone | ASL / GCF Control Module | Sense, calibrate, broadcast, sustain | Central ring rZ; driver + sense circuits |

Prototype P0 minimum configuration: 2 compute domains (A+B), 1 memory domain (C), 1 I/O/control domain (D), 4 outer rings, 1 central ring, 1 Rx per domain. Dummy dies or FPGA substitutes are acceptable for validation.

# 3. Minimum Third-Party Chiplet Interface

ASL does not modify compute datapaths. Each hosted chiplet requires only a small control endpoint:

| Interface signal | Direction | Function | Notes |
| --- | --- | --- | --- |
| RESET_IN | Platform → Chiplet | Hard or soft reset | Driven via ASL broadcast or copper fallback |
| POWER_GATE_IN | Platform → Chiplet | Power-state control | Emergency throttle from OBS |
| CLK_GATE_IN | Platform → Chiplet | Clock gating control | Barrier release, epoch marker |
| THERMAL_ALERT_OUT | Chiplet → Platform | Over-temperature signal | Input to ASL sense / OBS |
| CONGESTION_OUT | Chiplet → Platform | Queue / load status | Input to OBS policy |
| HEALTH_STATUS_OUT | Chiplet → Platform | Operational state | Heartbeat to ASL monitor |
| EMRG_THROTTLE_IN | Platform → Chiplet | Emergency reduce throughput | OBS rescue command via ASL |
| ASL_RX_IN | Interposer → Chiplet | Field broadcast decode | Receiver front-end output |
| CTRL_FALLBACK_IN | Copper bus → Chiplet | Redundant control | Standard sideband / I²C-like |

Receiver front-end (ASL_RX_IN) requires: envelope detector + comparator (~0.09mm² estimated), stub length ≤3mm to control entry point, supply isolated from compute core.

# 4. Physical Placement Specification

## 4.1 Ring geometry (F3 interposer/RDL layer)

| Parameter | Specification | Source | Status |
| --- | --- | --- | --- |
| Outer ring geometry | Square, ~3×3mm per domain | FastHenry abstract → physical translation | Re-extract required |
| Central ring geometry | Square, ~2×2mm at central zone perimeter | FastHenry abstract translation | Re-extract required |
| Trace width | 0.1 mm | FastHenry Phase 2 extraction | Verified |
| Trace thickness | 0.02 mm (20µm) | FastHenry Phase 2 extraction | Verified |
| Metal layer | Dedicated ASL control layer (RDL-3 in stack) | Floorplan v0.1 | Proposed |
| Keep-out clearance | 0.5 mm around ring trace | Estimated | Verify with EM rule |
| Domain size (each) | ~8×8mm within 32×32mm package | Floorplan v0.1 | Schematic |
| Vertical separation (ring to chiplet) | 0.2 mm (z_rx) | FastHenry A4b | Consistent |

## 4.2 Layer stack assignment

| Layer | Content | ASL interaction |
| --- | --- | --- |
| Chiplet / HBM die | Compute and memory dies | Receiver front-end bumps at base of each die |
| Bump / micro-bump | Die-to-interposer connection | ASL_RX_IN and control signals routed here |
| RDL upper | Local Rx connections, control stubs (≤3mm) | Short stubs from Rx front-end to chiplet control entry |
| RDL control (ASL layer) | 4 outer rings + 1 central ring + driver/sense | PRIMARY ASL LAYER; isolated from high-speed |
| RDL lower / interposer | Main high-speed interconnect, Cu data backbone | No ASL signals; ground return for ASL layer |
| Power / ground | PDN, power delivery | Ground plane for ASL return path |
| Package substrate | Structural, thermal, ball grid | Thermal path below power layer |

## 4.3 Sensitive-zone keep-out

| Sensitive element | Typical location | ASL keep-out rule |
| --- | --- | --- |
| SerDes / PHY | Domain C or D edge, near package I/O | Minimum 2mm from ring edge; verify coupling < M_sx 36pH baseline |
| PLL / Clock generator | Domain D, analog corner | No ring within 3mm; verify V_sx ratio ≤1.10 (Phase 2c-R2 gate) |
| HBM PHY | Domain C near HBM interface | Ring must not cross PHY boundary; validated Gamma_worst ≥2.875 |
| Power delivery analog | Central zone boundary | Driver circuit at zone perimeter only; not inside PDN analog core |

# 5. Two-Fabric Operation Specification

## 5.1 Data fabric (not ASL)

- Tensor payload, HBM traffic, coherent CPU–GPU data

- Large-volume chiplet-to-chiplet transfers

- Cache or memory consistency traffic

- Technology: UCIe, proprietary SerDes, future photonics

- NOT managed by ASL; ASL does not observe or modify data fabric

## 5.2 ASL control fabric

- Reset, wake-up, power-state transitions

- Barrier / epoch markers (high fan-out, low data volume)

- Thermal alert, congestion notification, health heartbeat

- Emergency throttle, OBS rescue command

- Calibration, field-level sensing, ASL mode transitions

- Low-rate broadcast (health status, epoch, telemetry) at ≤250 Mb/s per region

Separation principle: Data fabric maximizes bandwidth. ASL control fabric maximizes control reachability and resilience. They share the package but occupy different metal layers and never compete for the same resource.

# 6. Four Operating States

| State | Trigger | ASL action | Data fabric | Copper fallback |
| --- | --- | --- | --- | --- |
| NORMAL | All domains healthy, no alert | Sensing + calibration + low-rate broadcast (duty ≤2%) | Full operation | Standby |
| DEGRADED | One domain field drops 20–30% | Activate central ring boost (50–100mA, ≤50ns) Maintain R_rx ≥0.90 | Full operation | On standby |
| FAULT | Domain field drops >30% or Rx fails | Central boost (100–125mA) + switch to copper fallback for affected domain | Full operation | Active for affected domain |
| RECOVERY | Field restored or fault cleared | Reduce central ring, verify R_rx, resume low-rate broadcast, log event | Full operation | Remain active until stable |

# 7. Prototype P0 Measurable Gates

| Gate | Metric | Target | Method |
| --- | --- | --- | --- |
| P0-G1 | Field coupling Gamma_worst (square rings) | Gamma ≥2.0 | FastHenry re-extract with square geometry |
| P0-G2 | Fault-period R_rx (30% degradation, 100mA) | R_rx ≥0.90 | Full matrix solve (Phase 2c-R2 method) |
| P0-G3 | Sensitive-zone ratio | V_sx/V_sx_nom ≤1.10 | Same coupled solve |
| P0-G4 | Central ring current buildable | I(t=20ns) ≥90% of target at V≤0.8V | Phase 2c-R2 dynamic model |
| P0-G5 | ASL control broadcast bandwidth | 250 Mb/s / region achievable | Phase B3 receiver BW model |
| P0-G6 | Net area (neutral scenario) | Recovered ≥1.5× added | Zone Audit v0.2 neutral case (re-run after dedup) |
| P0-G7 | Normal-mode power overhead | ≤0.81mW (10% of 8.1mW Phase C2 saving) | Full circuit power estimate (Phase 2c-R2) |
| P0-G8 | Chiplet interface overhead | Rx frontend ≤0.09mm², stub ≤3mm | Architectural estimate |

# 8. Critical Open Items Before Layout Commitment

| Priority | Item | Blocker for |
| --- | --- | --- |
| P1 — Critical | Square-ring FastHenry Z-matrix re-extraction | All M, Gamma, R_rx numbers |
| P1 — Critical | Deduplicated Zone Audit (keepout ∩ broadcast tree area) | Net area claim |
| P2 — High | EM rule verification: 100mA in 0.1×0.02mm RDL trace | Current density and electromigration |
| P2 — High | Ground plane return path modeling | Full-wave field validation |
| P2 — High | Driver circuit design (NMOS boost, gate drive, slew) | P0 hardware prototype |
| P3 — Medium | Chiplet interface IP block (Rx frontend + decoder) | Third-party chiplet integration |
| P3 — Medium | OBS–ASL command interface protocol definition | GCF policy layer integration |
| P3 — Medium | OpenEMS full-wave validation of square-ring geometry | Physics upgrade from quasi-static |

# 9. Approved Platform Summary Statement

The ASL-Enabled AI Chiplet Platform v0.1 provides a package-level control and resilience fabric that hosts third-party AI compute chiplets via a minimal control interface (reset, power-gate, health, ASL Rx endpoint) without modifying compute datapaths. A five-zone interposer/RDL layout supports active field sustainment for up to 30% outer-ring degradation (R_rx ≥0.90, 100mA boost, V≤0.8V), low-rate broadcast at ≤250Mb/s per region, and operates as a parallel control fabric alongside any high-speed data interconnect. This specification is model-supported. Square-ring FastHenry re-extraction and EM rule verification are required before layout commitment.

ASL-Enabled AI Chiplet Platform Spec v0.1 — Confidential — Evidence: FastHenry + coupled-circuit + toy system | No silicon validation